Basic

- 1st small unit Neno meter

- 2nd small unit micrometer			`

- 1 micrometer 					1000 neno meter

- 1 mm 						1000 micrometer

- 1 meter 					39.37 inch

- 1 meter						1000 mm

- 1 meter						100 cm

- 1 cm 						10 mm

- 1 inch 						2.54 cm

- 1 inch 						25.4 mm

- 1 feet 						12 inch

- 1 feet 						30.48 cm

- 1 feet 						304.8 mm

- 1 Gaj 						3 feet

- 1 Gaj 						36 inch

- 1 kilometer 					1000 meter

CNC – Computer Numerical control

Definition of CNC

CNC is a advance manufacturing method that automates the control movement and precision of the machine tool, through the use of per-program computer software.

Type of CNC

CNC Machine

Two axis                                                                                   More than two axis

( x & z)                                                                                                 ( x, y, z, a, b……..)

CNC Turning                    CNC Wire Cut                     CNC Milling      CNC Grinding    CNC Hobbing CNCPlasma

VMC                                                                                HMC

( Vertical milling center )                                             (  Horizontal milling center )

Advantages

- Reduce cycle time & product cost

- Volume of production is very high

- Surface can be easily machined

- Flexibility in job change

- Automatic tool change

- Higher productivity & accuracy

- Better Quality, less paper work

- Operator safety

Working of CNC Turning Machine

The metal cutting from round surface or round job by tool known as CNC working

Operations

- OD Turning

- ID Turning

- Face Turning

- Taper Turning

- Drilling

- Chamfering

- Threading

- Radius

- Knurling

Main Parts of CNC Turning Machine

- Head Stock

- Tail Stock

- Turret

Head Stock

(Parts)

- Servo motor - It is used for rotating the spindle .

- Spindle - It is used for rotating the job.

- Draw Bar - It is operated by hydraulic pressure & used for opening & closing of jaws.

- Chuck - chuck is used for the holding the jaws . It is fitted with spindle & rotate by it.                                                                            Chuck are two type -  3 jaws chuck  , 4 jaws chuck

- Jaws-  Jaws are used for holding the job.

Jaws are two type – (1) Hard Jaws

(2) Soft Jaws

(2)  Tailstock - It is used to provide end support to longer job.

(3) Turret - Turret is used for holding the tool & support against cutting force during cutting.

Other Parts of CNC Machine

- Lubrication oil tank

- Hydraulic oil tank

- Coolant  water tank

- PLC Panel  ( Programmable Logic Control )

- Safety guard door

- Control Panel

Control Panel & Operator panel

There are seven operating modes used for different function

- Auto mode

- Jog Mode

- Handle Mode

- Single Block

- Edit Mode

- MDI (Manual Data Input )

- Reference Return

CNC Operating Mode

Manual Mode                       Auto Mode                      Edit Mode                       MDI                           Ref. Return

Jog             Handle        Single Block         Auto

Coordinate System

Coordinate system is used for getting point to point movement of tool according to coordinate of points.

Type Of Coordinate

- Absolute Coordinate - The Value of any point will be taken with respect of center point.

- Incremental Coordinate -The Value of any point will be taken with respect of previous point

RPM - The revolution made by spindle in one minute is called RPM.

Cutting Speed - The amount of any material removed by the cutting tool in a rotating work piece called cutting speed

Cutting Speed (Vc) - πDN/ 1000   Meter /Minute

Feed - The Distance traveled by the cutting tool in one revolution of the spindle of the machine is called feed.

Feed - mm/Revolution

Cutting Tool - A device that remove metal from any metal surface is called a cutting tool

Type of cutting tool

- Single point cutting tool - The Tool which has only one point of cutting is called single point cutting tool

Example - CNC Turning Insert (OD & ID)

- Multipoint cutting tool - The Tool which has multiple cutting points is called multipoint cutting tool

Example - Milling  cutter , Drill , Remer.

Type Of Insert

- Negative Insert

- Positive Insert

| Negative Insert | Positive Insert |  |
| --- | --- | --- |
| For Tool Holder | For Boring Bar |  |
| SNMG | SCMT | 90® |
| TNMG | TCMT | 60® |
| WNMG | WCMT | 80® |
| CNMT | CCMT | 80® |
| DNMG | DCMT | 55® |
| VNMG | VCMT | 35® |

C                    C                      M                    T                          09                     02                     03

D                   N                     M                     G                         16                     04                     08

Insert           Insert          Tolerance            Top of            Insert cutting     Insert             Corner

Shape           Angle                                           Surface        edge length       thickness         radius

G S Category

|  | GSA | GSB | GSC |
| --- | --- | --- | --- |
| Program Inch me | G20 | G20 | G70 |
| Threading | G32 | G33 | G33 |
| RPM | G50 | G92 | G92 |
| Feed mm/minute | G98 | G94 | G94 |
| Feed mm/revolution | G99 | G95 | G95 |
| Pocket cutting | G90 | G77 | G20 |
| Threading | G92 | G78 | G21 |
| Facing | G94 | G79 | G24 |
| Rapid travel | G00 | G00 | G00 |
| Liner Interpolation | G01 | G01 | G01 |
| Reference return | G28 | G28 | G28 |

G Codes

These are preparatory function codes & used for axis motion

Axis moment codes

G00     This code is used for tool positioning & free moment

- Format – G0 X……..Z………;

G01     This code is used for liner movement of axis

- Format – G01 X……… Z…………F………;

G02     This code is used for circular clock wise movement of any axis

- Format – G02 X……..Z……….R………F……..;

G03      This code is used for circular anticlockwise movement of any axis

- Format – G03 X………Z………..R……..F……..;

G28     This code is used for homing in any axis

- Format – G28 U0.0 W0.0 ;

Dwell time

- G04 X……… (Time in second)

- G04 U ………. (Time in second)

Inch/ mm mode setting

- G20 – Program in inch.

- G21 – Program in mm.

Reference return

- G28 –  1st reference point

- G30 – 2nd reference point

Skip Function

- G31 – skip function

Thread Cutting

- G32 – thread cutting

- G33 -  thread cutting

Tool Nose Radius

- G40 – Tool nose radius compensation

- G41 - Tool nose radius compensation left

- G42 – Tool nose radius compensation right

Canned Cycle

- G70 – Finishing cycle of 71, 72 ,73

- G71 – Stock removal of Turning cycle

- G72/G94 – Stock removal of facing cycle

- G73 – Patten repeating cycle

- G74 – Drilling cycle

- G75 – Grooving cycle

- G76 – multiple threading cycle

- G77/G90 – Pocket cutting cycle & turning

- G78 – Single threading cycle

- G79 – Pocket cutting in facing

- G80 – Canned cycle cancel

Mode setting

- G90 – Absolute Programming

- G91 – Incremental Programming

- G92/G50 – max. spindle rpm

- G94 – Feed mm/minute

- G95 – Feed mm/revolution

- G96 – Constant surface speed control

- G97 – constant surface speed control cancel

- G98 – feed mm/minute

- G99 – feed mm/revolution

M Code

M Code is switch code of CNC machine

- M00 – Program stop

- M01 – Optional stop

- Mo2 – Program end

- M03 – Spindle start (C.C.W)

- M04 – Spindle start (C.W)

- M05 – Spindle stop

- M06 – Tool change in VMC

- M07 -  Coolant on

- M08 – Coolant on

- M09 – Coolant off

- M10 – Chuck clamp

- M11 – Chuck unclamp

- M17 – Turret rotation forward

- M18 – Turret rotation reverse

- M19 – Spindle orientation on

- M20 – Spindle orientation off

- M30 – Program end ,reset

- M98 – Sub program call

- M99 – Sub program end & return

Offset

To make a new point in software or shifting any point in software from its actual position known as offset

Type of offset

GEOMETRY OFFSET - Geometry offset is used for making job origin or job center at a distance from machine home position

WEAR OFFSET – Wear offset is used for shifting the tool from its actual position by giving the value in software .In CNC turning wear offset is used in X&Z axis.

WORKSHIFT OFFSET – Work shift offset is used for shifting the job origin from its actual position by giving the value in software

Program Scan – CNC Machine scan the program sequencely  in block from top block to bottom block .

Program Header

- Program No. OR Name                          O0001 ;

- Sequence no.                                            N1;

- Home Position                                         G28 U0.0 W0.0 ;

- Tool & Offset selection                          T0101;

- Spindle Start  RPM With Direction       G97 M04 S1000 ;

- Tool take a safe zone position             G0 X150.0 Z50.0 ;

- Coolant ON                                              M08 ;

Program Footer

- Tool take a safe zone position               G0 X150.0 Z50.0 ;

- Coolant off                                                M09 ;

- Machine home position                         G28 U0.0 W0.0 ;

- Spindle Stop                                             M05 ;

- Program End & Reset                              M30 ;